# Lab 12 Setting up Paramiko for SSH Communication

Before using Paramiko in Python scripts, it’s essential to install the library and set up the development environment. Paramiko can be installed via pip, the Python package manager, using the following command:

pip install paramiko

Once installed, developers can import the Paramiko module in their Python scripts and start using its classes and functions to establish SSH connections and interact with remote devices.

Establishing Secure Connections to Network Devices

Paramiko allows developers to establish secure SSH connections to network devices, servers, and other systems. The process involves creating a Paramiko SSH client object, specifying connection parameters such as the hostname, port, and authentication credentials, and then initiating the connection.

import paramiko

# Create SSH client

client = paramiko.SSHClient()

# Set policy to automatically add hosts to known hosts file

client.set_missing_host_key_policy(paramiko.AutoAddPolicy())

# Connect to the remote host

client.connect(

hostname="172.16.10.12",

username="admin",

password="cisco",

look_for_keys=False,

allow_agent=False,

)

# Perform operations on the remote host

# Close the connection

client.close()

Executing Commands and Handling Responses Asynchronously

Once a secure SSH connection is established, developers can execute commands on the remote host and handle the command output asynchronously using Paramiko. This allows for efficient execution of multiple commands and parallel processing of command responses.

import paramiko

import time

# Create SSH client

client = paramiko.SSHClient()

# Set policy to automatically add hosts to known hosts file

client.set_missing_host_key_policy(paramiko.AutoAddPolicy())

# Connect to the remote host

client.connect(

hostname="172.16.10.12",

username="admin",

password="cisco",

look_for_keys=False,

allow_agent=False,

)

# Open an interactive SSH session

ssh_client = client.invoke_shell()

# Send command

ssh_client.send("sh ip int bri\n")

# Wait for the command to be finished

time.sleep(3)

# Receive and process command output

output = ssh_client.recv(65000)

print(output.decode("ascii"))

# Close the SSH session

ssh_client.close()

# Close the connection

client.close()

R1#sh ip int bri

Interface                  IP-Address      OK? Method Status                Protocol

FastEthernet0/0            172.16.10.12    YES NVRAM  up                    up

FastEthernet0/1            unassigned      YES NVRAM  administratively down down

R1#

Handling Exceptions and Error Scenarios Gracefully

In network automation and remote system management, it’s crucial to handle exceptions and error scenarios gracefully to ensure the robustness and reliability of the application. Paramiko provides mechanisms for catching and handling exceptions that may occur during SSH communication, such as authentication errors, connection timeouts, and network failures.

import paramiko

import time

# Create SSH client

try:

client = paramiko.SSHClient()

# Set policy to automatically add hosts to known hosts file

client.set_missing_host_key_policy(paramiko.AutoAddPolicy())

# Connect to the remote host

client.connect(

hostname="172.16.10.12",

username="admin",

password="cisco",

look_for_keys=False,

allow_agent=False,

)

# Open an interactive SSH session

ssh_client = client.invoke_shell()

# Send command

ssh_client.send("sh ip int bri\n")

# Wait for the command to be finished

time.sleep(3)

# Receive and process command output

output = ssh_client.recv(65000)

print(output.decode("ascii"))

# Close the SSH session

ssh_client.close()

# Close the connection

client.close()

except paramiko.AuthenticationException:

print("Authentication failed. Please verify your credentials.")

By effectively handling exceptions and error scenarios, developers can build resilient and fault-tolerant Python applications for secure SSH communication using Paramiko.

Advanced Example: Creating Loopback Interfaces

In the next example, we employ a for loop and the range() function to create loopback interfaces on a Cisco router.

import paramiko

import time

# Create an SSH client instance

client = paramiko.SSHClient()

client.set_missing_host_key_policy(paramiko.AutoAddPolicy())

# Connect to the remote device

client.connect(

hostname="172.16.10.12",

username="admin",

password="cisco",

look_for_keys=False,

allow_agent=False,

)

# Start an interactive shell

ssh_client = client.invoke_shell()

print("###### Creating loopback interfaces ######")

# ssh_client.send("cisco\n")

ssh_client.send("conf ter\n")

# Use a for loop and range() to create loopback interfaces

for interface_number in range(0, 1):

ssh_client.send(f"int lo {interface_number}\n")

ssh_client.send(f"ip address 1.1.1.{interface_number} 255.255.255.255\n")

# Wait for the configurations to take effect

time.sleep(1)

ssh_client.send("end\n")

ssh_client.send("show ip int brief\n")

# Wait for the output and retrieve it

time.sleep(3)

output = ssh_client.recv(65000)

print(output.decode("ascii"))

# Close the SSH connection

client.close()

###### Creating loopback interfaces ######

R1#conf ter

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)#int lo 0

R1(config-if)#ip address 1.1.1.0 255.255.255.255

R1(config-if)#end

R1#show ip int brief

Interface                  IP-Address      OK? Method Status                Protocol

FastEthernet0/0            172.16.10.12    YES NVRAM  up                    up

FastEthernet0/1            unassigned      YES NVRAM  administratively down down

Loopback0                  1.1.1.0         YES manual up                    up

R1#

This advanced example demonstrates the flexibility of Paramiko for configuring network devices programmatically. The script showcases the creation of loopback interfaces, providing a practical illustration of how automation can simplify repetitive tasks.

Connecting to Multiple Devices

Expanding our horizons, the following example demonstrates using a for loop to connect to multiple devices sequentially.

import paramiko

import time

client = paramiko.SSHClient()

client.set_missing_host_key_policy(paramiko.AutoAddPolicy())

# For loop and range() function to connect to multiple devices

for device in range(11, 12):

host = "172.16.10." + str(device)

print("\n##### Connecting to the device " + host + " #####")

client.connect(

hostname=host,

username="admin",

password="cisco",

look_for_keys=False,

allow_agent=False,

)

ssh_client = client.invoke_shell()

ssh_client.send("show ip int brief\n")

time.sleep(3)

output = ssh_client.recv(65000)

print(output.decode("ascii"))

client.close()

##### Connecting to the device 172.16.10.11 #####

**************************************************************************

* IOSv is strictly limited to use for evaluation, demonstration and IOS  *

* education. IOSv is provided as-is and is not supported by Cisco's      *

* Technical Advisory Center. Any use or disclosure, in whole or in part, *

* of the IOSv Software or Documentation to any third party for any       *

* purposes is expressly prohibited except as otherwise authorized by     *

* Cisco in writing.                                                      *

**************************************************************************

SW1#show ip int brief

Interface              IP-Address      OK? Method Status                Protocol

GigabitEthernet0/0     unassigned      YES unset  up                    up

GigabitEthernet0/1     unassigned      YES unset  up                    up

GigabitEthernet0/2     unassigned      YES unset  down                  down

GigabitEthernet0/3     unassigned      YES unset  down                  down

GigabitEthernet1/0     unassigned      YES unset  down                  down

GigabitEthernet1/1     unassigned      YES unset  down                  down

GigabitEthernet1/2     unassigned      YES unset  down                  down

GigabitEthernet1/3     unassigned      YES unset  down                  down

GigabitEthernet2/0     unassigned      YES unset  down                  down

GigabitEthernet2/1     unassigned      YES unset  down                  down

GigabitEthernet2/2     unassigned      YES unset  down                  down

GigabitEthernet2/3     unassigned      YES unset  down                  down

GigabitEthernet3/0     unassigned      YES unset  down                  down

GigabitEthernet3/1     unassigned      YES unset  down                  down

GigabitEthernet3/2     unassigned      YES unset  down                  down

GigabitEthernet3/3     unassigned      YES unset  down                  down

Loopback0              1.1.1.0         YES manual up                    up

Vlan1                  172.16.10.11    YES NVRAM  up                    up

SW1#

##### Connecting to the device 172.16.10.12 #####

R1#show ip int brief

Interface                  IP-Address      OK? Method Status                Protocol

FastEthernet0/0            172.16.10.12    YES NVRAM  up                    up

FastEthernet0/1            unassigned      YES NVRAM  administratively down down

Loopback0                  1.1.1.0         YES manual up                    up

R1#

This script demonstrates the scalability of Paramiko, allowing network engineers to connect and interact with multiple devices seamlessly.

Connecting to Multiple Devices with a List

In this example, we use a for loop and a list to connect to multiple devices. This approach provides greater flexibility and ease of maintenance.

import paramiko

import time

username = "admin"  # username

password = "cisco"  # password

# IP list for network devices

devices = ["172.16.10.11", "172.16.10.12"]

# For loop and list to connect to multiple devices

for device in devices:

print("\n #### Connecting to the device " + device + " ####\n")

client = paramiko.SSHClient()

client.set_missing_host_key_policy(paramiko.AutoAddPolicy())

client.connect(

hostname=device,

port=22,

username=username,

password=password,

look_for_keys=False,

allow_agent=False,

)

ssh_client = client.invoke_shell()

ssh_client.send("config ter\n")

# For loop and range() function to create loop back interface

for num in range(2, 5):

ssh_client.send("int loop " + str(num) + "\n")

ssh_client.send("ip address 1.1.1." + str(num) + " 255.255.255.255\n")

time.sleep(1)

ssh_client.send("end\n")

# ssh_client.send("term length 0\n")

ssh_client.send("show ip int brief\n")

time.sleep(3)

output = ssh_client.recv(65000)

print(output.decode("ascii"))

client.close()

# #### Connecting to the device 172.16.10.11 ####

# **************************************************************************

# * IOSv is strictly limited to use for evaluation, demonstration and IOS  *

# * education. IOSv is provided as-is and is not supported by Cisco's      *

# * Technical Advisory Center. Any use or disclosure, in whole or in part, *

# * of the IOSv Software or Documentation to any third party for any       *

# * purposes is expressly prohibited except as otherwise authorized by     *

# * Cisco in writing.                                                      *

# **************************************************************************

SW1#config ter

Enter configuration commands, one per line.  End with CNTL/Z.

SW1(config)#int loop 2

SW1(config-if)#ip address 1.1.1.2 255.255.255.255

SW1(config-if)#int loop 3

SW1(config-if)#ip address 1.1.1.3 255.255.255.255

SW1(config-if)#int loop 4

SW1(config-if)#ip address 1.1.1.4 255.255.255.255

SW1(config-if)#end

SW1#show ip int brief

Interface              IP-Address      OK? Method Status                Protocol

GigabitEthernet0/0     unassigned      YES unset  up                    up

GigabitEthernet0/1     unassigned      YES unset  up                    up

GigabitEthernet0/2     unassigned      YES unset  down                  down

GigabitEthernet0/3     unassigned      YES unset  down                  down

GigabitEthernet1/0     unassigned      YES unset  down                  down

GigabitEthernet1/1     unassigned      YES unset  down                  down

GigabitEthernet1/2     unassigned      YES unset  down                  down

GigabitEthernet1/3     unassigned      YES unset  down                  down

GigabitEthernet2/0     unassigned      YES unset  down                  down

GigabitEthernet2/1     unassigned      YES unset  down                  down

GigabitEthernet2/2     unassigned      YES unset  down                  down

GigabitEthernet2/3     unassigned      YES unset  down                  down

GigabitEthernet3/0     unassigned      YES unset  down                  down

GigabitEthernet3/1     unassigned      YES unset  down                  down

GigabitEthernet3/2     unassigned      YES unset  down                  down

GigabitEthernet3/3     unassigned      YES unset  down                  down

Loopback0              1.1.1.0         YES manual up                    up

Loopback2              1.1.1.2         YES manual up                    up

Loopback3              1.1.1.3         YES manual up                    up

Loopback4              1.1.1.4         YES manual up                    up

Vlan1                  172.16.10.11    YES NVRAM  up                    up

SW1#

#### Connecting to the device 172.16.10.12 ####

R1#config ter

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)#int loop 2

R1(config-if)#ip address 1.1.1.2 255.255.255.255

R1(config-if)#int loop 3

R1(config-if)#ip address 1.1.1.3 255.255.255.255

R1(config-if)#int loop 4

R1(config-if)#ip address 1.1.1.4 255.255.255.255

R1(config-if)#end

R1#show ip int brief

Interface                  IP-Address      OK? Method Status                Protocol

FastEthernet0/0            172.16.10.12    YES NVRAM  up                    up

FastEthernet0/1            unassigned      YES NVRAM  administratively down down

Loopback0                  1.1.1.0         YES manual up                    up

Loopback2                  1.1.1.2         YES manual up                    up

Loopback3                  1.1.1.3         YES manual up                    up

Loopback4                  1.1.1.4         YES manual up                    up

R1#

This script introduces a more modular approach, where devices are managed through a list, providing simplicity and ease of maintenance.

Conclusion

Paramiko plays a pivotal role in network automation, empowering network engineers to streamline and automate various tasks efficiently. By leveraging Paramiko for secure SSH communication with networking devices, network engineers can enhance network operations’ efficiency and reliability.

The examples provided serve as foundational knowledge for creating customized automation workflows tailored to specific network requirements. Embrace the power of Paramiko and Python to adapt and thrive in the dynamic landscape of network management.